WEEKLY PM CHECKLIST

Instructions:

- Fill out this form to document and record Weekly PM per QMS P-19 Procedure

- Submit completed form to a supervisor or plant manager.

_____________________________________________________________________________________________

| Equip ID | Press | Hydraulic Configuration | Plenum | Class |
| --- | --- | --- | --- | --- |
| RF-EQ-001 | Dake 300T Single-Ram Press | Single ram, 300 ton, ~1965 | Stand-alone | A |
| RF-EQ-002 | Large Press #1 – 9-Ram | 9× 3,000 PSI rams | Shared w/ Press #2 | A |
| RF-EQ-003 | Large Press #2 – 9-Ram | 9× 3,000 PSI rams | Shared w/ Press #1 | A |
| RF-EQ-004 | Small Press #1 – 5-Ram | 5× 3,000 PSI rams | Independent | A |
| RF-EQ-005 | Small Press #2 – 5-Ram | 5× 3,000 PSI rams | Independent | A |

1.0 Weekly Preventive Maintenance  (All Presses)

30–45 minutes per press. Scheduled weekly. Not optional for Class A equipment.

1.1 ALL PRESSES — Weekly Checks

| ☐ Tighten visible hose fittings — apply witness marks after tightening | ☐ Inspect ram rods for scoring, pitting, or chrome damage |
| --- | --- |
| ☐ Check hose abrasion points — rubber-on-metal frame contact | ☐ Filter ΔP gauge or spin-on filter visual inspection |
| ☐ Inspect breather caps — clean or replace if plugged | ☐ Verify emergency stop function on each press |
| ☐ Verify pressure relief valve — not tampered with, seated properly | ☐ Log oil tank temperature during production run |
| ☐ Inspect pump coupling — wear or misalignment signs | ☐ Clean around platen interfaces and press frame |

5.2 LARGE PRESSES — Shared Plenum Additional Weekly

| ☐ Inspect suction strainer access — no restriction signs | ☐ Inspect return manifold for vibration cracks at weld joints |
| --- | --- |
| ☐ Pump coupling inspection — both presses, wear and alignment | ☐ Motor bearing temperature — compare Press #1 vs Press #2 |

| Technician: ____________________ | Date: ____________ | Time: ____________ | Supervisor: ____________________ |
| --- | --- | --- | --- |

Notes / Issues Found:

Escalation Rules

| Trigger | Immediate Action | Follow-Up Required |
| --- | --- | --- |
| Oil below minimum on any Class A press | Lock out press. Halt production on affected unit. | Notify supervisor. WO opened. No restart until oil topped and cause identified. |
| Shared plenum oil low (Large Presses) | Lock out BOTH Large Presses simultaneously. | Notify VP Operations. Root cause documented before restart. |
| Cavitation noise on pump | Lock out press. Do not run. | Maintenance inspection within 2 hours. WO opened. |
| Missed Class A PM (any reason) | Written explanation required same day. | Root cause logged. Corrective action documented. VP Operations review. |
| Burning oil smell (Hot Oil Boiler) | Immediate shutdown + lockout. | No restart without VP Operations authorization. |
| SMARTLINK disconnected (Compressor) | Reconnect immediately. | Notify Atlas Copco if not resolved within 24 hrs. |

| - Revision | - Reason | - Revised by | - Approved by | - Date |
| --- | --- | --- | --- | --- |
| - Original | - Initial Release | - J Robbins | - J Probst | - 4/21/2026 |
| - A |  |  |  |  |
| - B |  |  |  |  |