- Purpose

This document establishes the Preventive Maintenance (PM) system for all production equipment, material handling equipment, utilities, and facility systems at RubberForm Recycled Products, LLC.

- Scope

This procedure defines mandatory PM schedules, accountability requirements, escalation rules, and vendor PM tracking for all registered equipment line items.

- References

  - ISO 9001

  - GMP Guidelines

  - Company Quality Management System (QMS)

- Definitions and Criticality Classifications

| Class | Definition | Downtime Impact | PM Enforcement |
| --- | --- | --- | --- |
| A | Revenue / Safety Critical — stops production or creates immediate safety risk | Immediate revenue loss | Mandatory. Missed PM = escalation + written explanation |
| B | Operational Impact — disrupts flow; does not immediately stop production | Hurts throughput within 24–48 hrs | Tracked monthly. Miss allowed with explanation. |
| C | Low Risk — tools, peripherals, shop equipment | Minor — workaround usually available | Quarterly / annual batch inspections |

- Responsibilities

  - Maintenance Manager has primary responsibility for executing this procedure including daily, weekly, and monthly PM checklists, work order submission and spare parts orders.

  - Plant Manager has secondary responsibility to ensure PM activities are planned and executed in a manner which minimally affects the production schedule.

  - Director of Engineering is responsible for supporting PM activities that may require higher-level and complex analysis, trouble shooting, root cause identification, or design alterations.

  - VP of Operations has oversight and management responsibility for this procedure.  This shall be accomplished by monitoring and reporting various KPI’s (key performance indicators).

- Procedure

  - Conduct daily, weekly and monthly PM checks.  These are documented by using appropriate checklists published in the QMS and by logging into RFRP QMS Tracker (https://qms-mx-system-production.up.railway.app)

    - Daily checks: Initialed paper checklist posted at each press group — signature + date required

    - Weekly/monthly: Logged in QMS web application (RFRP QMS Tracker) and/or shared drive

    - Missed PM: Written explanation required — root cause + corrective action.

    - Vendor PM: Completion certificates filed within 48 hours of vendor visit

  - Use RFRP QMS Tracker (https://qms-mx-system-production.up.railway.app) to document daily, weekly and monthly PM as appropriate.  This app shall be the primary means for tracking, managing and measuring the performance of the PM system.

  - PM compliance is reviewed weekly by VP Operations. Monthly KPI report generated from QMS web application. Results posted on production floor compliance board.

| KPI Metric | Target | Notes |
| --- | --- | --- |
| PM Compliance — Class A (Internal) | ≥ 95% | Presses, boilers, extruders, mixers |
| PM Compliance — Compressors (Atlas Copco) | 100% | Vendor-managed annual PM |
| PM Compliance — Forklifts (JIT) | 100% | Vendor-managed 180-day PM |
| PM Compliance — HVAC/Gas (EMCOR) | 100% | Vendor-managed quarterly PM |
| PM Compliance — Class C Tools | ≥ 85% | Shop tools, peripherals |
| Unplanned Downtime | < 8 hrs/month | Track per machine; log root cause |
| Open Work Orders | < 5 | Escalate if Critical WO > 48 hrs open |
| Critical WOs Overdue | 0 | Zero tolerance |

Compliance % formula: (Completed PMs ÷ Scheduled PMs) × 100. Vendor-managed PMs excluded from internal compliance calculation.

- Supporting Documents

  - Use Form F-19xxxxx as appropriate.  See QMS Forms

  - Use RFRP QMS Tracker (https://qms-mx-system-production.up.railway.app)

  - Use other appropriate QMS procedures and forms to ensure PM activities are completed.

- Recordkeeping

  - Use RFRP QMS Tracker (https://qms-mx-system-production.up.railway.app) to document daily, weekly and monthly PM as appropriate.  This app shall be the primary means for tracking, managing and measuring the performance of the PM system.

  - Any paper forms may be filed as appropriate, or scanned and in Q:\QMS CURRENT COPY\MASTER\Records

- Revision History

  - Any modifications must be documented with a revision history, including changes made and the reason for the update.

  - Revised documents require review and approval before implementation.

  - Old versions must be archived and marked as "Obsolete."

| - Revision | - Reason | - Revised by | - Approved by | - Date |
| --- | --- | --- | --- | --- |
| - Original | - Initial Release | - J Robbins | - J Probst | - 4/21/2026 |
| - A |  |  |  |  |
| - B |  |  |  |  |