DAILY PM CHECKLIST

Instructions:

- Fill out this form to document and record Daily PM per QMS P-19 Procedure

- Submit completed form to a supervisor or plant manager.

_____________________________________________________________________________________________

| Equip ID | Press | Hydraulic Configuration | Plenum | Class |
| --- | --- | --- | --- | --- |
| RF-EQ-001 | Dake 300T Single-Ram Press | Single ram, 300 ton, ~1965 | Stand-alone | A |
| RF-EQ-002 | Large Press #1 – 9-Ram | 9× 3,000 PSI rams | Shared w/ Press #2 | A |
| RF-EQ-003 | Large Press #2 – 9-Ram | 9× 3,000 PSI rams | Shared w/ Press #1 | A |
| RF-EQ-004 | Small Press #1 – 5-Ram | 5× 3,000 PSI rams | Independent | A |
| RF-EQ-005 | Small Press #2 – 5-Ram | 5× 3,000 PSI rams | Independent | A |

1.0 Daily Preventive Maintenance  (All Presses)

Performed at start of shift, machine cold if possible. 10–15 minutes per press group. Non-negotiable for all Class A presses.

1.2 ALL PRESSES — Daily Checks

| ☐ Hydraulic oil level — record actual sight glass / dipstick reading | ☐ Visual leak check: hoses, fittings, manifolds, ram seals, floor below press |
| --- | --- |
| ☐ Oil condition — check for darkening (oxidation), foam (aeration), milky appearance (water ingress) | ☐ Pressure gauge at idle — compare to posted normal operating range |
| ☐ Pump noise on startup — listen for cavitation, whine, grinding | ☐ Pressure gauge under load — compare to posted spec |
| ☐ Slow pressure decay after cycle — ram drift = seal issue | ☐ Control panel / PLC — no warning lights or fault indicators |
| ☐ Area clean — no oil accumulation on floor or press base | ☐ Emergency stop — verify functional (test weekly minimum) |

⚠ LOCKOUT RULE: If oil is below minimum — LOCK OUT THE PRESS. No production. No 'we will top it later.' Notify supervisor immediately.

1.3 LARGE PRESSES #1 & #2 (RF-EQ-002/003) — Shared Plenum Additional Checks

| ☐ Shared plenum oil level — log ONCE per shift; document actual level for BOTH presses | ☐ Pump inlet line — no weeping or air entrainment signs |
| --- | --- |
| ☐ Return-line temperature by hand — hot = restriction or bypass issue | ☐ Motor amps — quick glance vs normal baseline |

⚠ SHARED PLENUM RULE: If shared plenum oil is low — BOTH Large Presses lock out simultaneously. One plenum = two presses. Shared failure is a double shutdown.

1.4 SMALL PRESSES #1 & #2 (RF-EQ-004/005) — Independent Plenum Checks

| ☐ Press #1 plenum oil level — record actual reading | ☐ Press #2 plenum oil level — record actual reading (independent; does not share) |
| --- | --- |
| ☐ Motor amps — clamp meter check vs normal baseline for each unit | ☐ Pump noise on each motor — cavitation or bearing noise |

DAKE 300T PRESS (RF-EQ-001) — Stand-Alone Additional Checks

| ☐ Oil level — old tank; notorious for slow leaks; check more closely | ☐ Ram seal seepage — wipe rod; mark any fresh weeping with marker |
| --- | --- |
| ☐ Pump noise on cold startup — cavitation signs indicate low oil or wear | ☐ Relief valve — not tampered with or bypassed |

| Technician: ____________________ | Date: ____________ | Time: ____________ | Supervisor: ____________________ |
| --- | --- | --- | --- |

Notes / Issues Found:

2.0 HOT OIL BOILER — DAILY

| ☐ Oil level — expansion tank sight glass reading | ☐ Supply temperature — log setpoint vs actual |
| --- | --- |
| ☐ Return temperature — log and compare to supply delta | ☐ Pump noise — cavitation or seal leak sounds |
| ☐ Visual leak — fittings, flanges, pump seal, boiler jacket | ☐ Burner ignition normal — no delayed ignition or misfires |
| ☐ Safety interlock indicators — no fault lights | ☐ Smell check — burning oil odor = immediate shutdown and lockout |

⚠ HOT OIL SAFETY: Burning oil smell = immediate shutdown. Do not restart without authorization from maintenance manager and VP Operations.

3.0 Air Compressors (RF-EQ-013 & 014) — Operator Daily Checks

Note: Full PM is vendor-managed by Atlas Copco Compressor Services (Quote 158395228, 5-yr agreement). Daily and weekly operator checks below are a CUSTOMER OBLIGATION under the agreement. Failure to perform these checks voids the warranty.

| ☐ Oil level — dipstick reading; log actual | ☐ Drain condensate from tank and dryer |
| --- | --- |
| ☐ Listen for abnormal sounds — knocking, air leaks, bearing noise | ☐ Pressure relief valve — no weeping at seat |
| ☐ Outlet air pressure — meets plant system requirements | ☐ Dryer / separator — no alarm indicators on SMARTLINK or panel |
| ☐ Check temperatures and pressures — within normal operating range | ☐ SMARTLINK connected — verify unit shows online in controller |

⚠ ATLAS COPCO CONTRACT REQUIREMENT: Customer is responsible for daily and weekly inspections. Inform Atlas Copco of any changes in operational conditions. SMARTLINK must remain connected continuously.

Escalation Rules

| Trigger | Immediate Action | Follow-Up Required |
| --- | --- | --- |
| Oil below minimum on any Class A press | Lock out press. Halt production on affected unit. | Notify supervisor. WO opened. No restart until oil topped and cause identified. |
| Shared plenum oil low (Large Presses) | Lock out BOTH Large Presses simultaneously. | Notify VP Operations. Root cause documented before restart. |
| Cavitation noise on pump | Lock out press. Do not run. | Maintenance inspection within 2 hours. WO opened. |
| Missed Class A PM (any reason) | Written explanation required same day. | Root cause logged. Corrective action documented. VP Operations review. |
| Burning oil smell (Hot Oil Boiler) | Immediate shutdown + lockout. | No restart without VP Operations authorization. |
| SMARTLINK disconnected (Compressor) | Reconnect immediately. | Notify Atlas Copco if not resolved within 24 hrs. |

| - Revision | - Reason | - Revised by | - Approved by | - Date |
| --- | --- | --- | --- | --- |
| - Original | - Initial Release | - J Robbins | - J Probst | - 4/21/2026 |
| - A |  |  |  |  |
| - B |  |  |  |  |