MONTHLY PM CHECKLIST

Instructions:

- Fill out this form to document and record Weekly PM per QMS P-19 Procedure

- Submit completed form to a supervisor or plant manager.

_____________________________________________________________________________________________

| Equip ID | Press | Hydraulic Configuration | Plenum | Class |
| --- | --- | --- | --- | --- |
| RF-EQ-001 | Dake 300T Single-Ram Press | Single ram, 300 ton, ~1965 | Stand-alone | A |
| RF-EQ-002 | Large Press #1 – 9-Ram | 9× 3,000 PSI rams | Shared w/ Press #2 | A |
| RF-EQ-003 | Large Press #2 – 9-Ram | 9× 3,000 PSI rams | Shared w/ Press #1 | A |
| RF-EQ-004 | Small Press #1 – 5-Ram | 5× 3,000 PSI rams | Independent | A |
| RF-EQ-005 | Small Press #2 – 5-Ram | 5× 3,000 PSI rams | Independent | A |

1.0  Monthly Preventive Maintenance  (All Presses)

1–2 hours per press group. Scheduled first week of each month.

1.2 ALL PRESSES — Monthly Checks

| ☐ Hydraulic oil sample — bottle and label; send quarterly to lab | ☐ Inspect relief valves — reseat if required |
| --- | --- |
| ☐ Inspect pump coupling — replace if worn | ☐ Inspect platen alignment indicators if present |
| ☐ Inspect electrical terminations in control cabinet | ☐ Verify pressure transducer vs mechanical gauge cross-check |
| ☐ Inspect all hydraulic hoses — flag any over 5 years old | ☐ Log all ram seal conditions — grade each: Good / Watch / Replace |
| ☐ Photograph any new leaks, scoring, or damage | ☐ Log total run hours if hour meter present |

6.2 QUARTERLY / SEMI-ANNUAL (Scheduled by VP Operations)

| ☐ Hydraulic oil analysis — ISO cleanliness, TAN, water content | ☐ Filter cartridge replacement (regardless of appearance) |
| --- | --- |
| ☐ Relief valve test and reseat | ☐ Electrical IR scan if available |
| ☐ Ram seal condition assessment — replace any flagged from monthly log | ☐ Full hose inspection — replace any flagged as age risk |

| Technician: ____________________ | Date: ____________ | Time: ____________ | Supervisor: ____________________ |
| --- | --- | --- | --- |

Notes / Issues Found:

Escalation Rules

| Trigger | Immediate Action | Follow-Up Required |
| --- | --- | --- |
| Oil below minimum on any Class A press | Lock out press. Halt production on affected unit. | Notify supervisor. WO opened. No restart until oil topped and cause identified. |
| Shared plenum oil low (Large Presses) | Lock out BOTH Large Presses simultaneously. | Notify VP Operations. Root cause documented before restart. |
| Cavitation noise on pump | Lock out press. Do not run. | Maintenance inspection within 2 hours. WO opened. |
| Missed Class A PM (any reason) | Written explanation required same day. | Root cause logged. Corrective action documented. VP Operations review. |
| Burning oil smell (Hot Oil Boiler) | Immediate shutdown + lockout. | No restart without VP Operations authorization. |
| SMARTLINK disconnected (Compressor) | Reconnect immediately. | Notify Atlas Copco if not resolved within 24 hrs. |

| - Revision | - Reason | - Revised by | - Approved by | - Date |
| --- | --- | --- | --- | --- |
| - Original | - Initial Release | - J Robbins | - J Probst | - 4/21/2026 |
| - A |  |  |  |  |
| - B |  |  |  |  |